/*!

* \file 	02_What_Is_Chan_SCCP.doc

* \brief 	An implementation of Skinny Client Control Protocol (SCCP)

* \author 	Diederik de Groot <dkgroot [at] talon.nl>

* \since	2009-11-12

* \brief 	Main Documentation

* \note 	This program is free software and may be modified and distributed under the terms of the GNU Public License.

* \version	$Revision: 1369 $

* \date	$date$

**/

/*!

* \file

* Contains extra comments for Doxygen.

*

* You could read the documentation from this file; but it would probably

* be easier to go to http://chan-sccp-b.sourceforge.net

*

**/

/*!

* \page 	what_sccp 		What is Chan_SCCP

*

* \section 	what_is_chan_sccp 	What is Chan_SCCP

*

* Chan_SCCP is a replacement Channel Driver for chan_skinny in the Asterisk Channel Driver Library. It delivers

* better performance, scalability, interoperability and functionality than either chan_skinny or chan_sip on a

* SCCP capable phone. It is still under heavy development but a stable release is also available for you to use.

*

* To read more about asterisk, you can find a nice introduction <a href="http://www.aosabook.org/en/asterisk.html">here</a>.

*

* \subsection 	what_sccp_ss1		SCCP in relation to SIP/IAX/MGCP

*

* There are several methods to communicate with a VOIP Server. Next to the well known SIP Protocol there are

* Several others like: MGCP, IAX, IAX2, SS7 and SCCP. The SCCP protocol is sometimes refered to as SKINNY.

*

* Several VOIP Phone Resellers have create phones that use the SCCP Protocol. The most well known at this moment

* is CISCO, but there are others like Kirk, Ixia, Symbol Techologies, SocketIP, Digium and Nokia.

*

* IPBlue markets a software phone (soft phone) which uses SCCP for signaling and emulates a Cisco 7960 hardware phone.

*

* \subsection 	what_sccp_ss2 		SCCP as a Protocol

*

* The Skinny Call Control Protocol a.k.a Signalling Connection Control Part (SCCP, Q.713, or short Skinny) is a

* proprietary network terminal control protocol originally developed by Selsius Corporation. The term "Skinny"

* reflects that SCCP is a simple and uncomplicated ("lightweight") protocol requiring relatively little computer

* processing.

*

* The SCCP technology is now owned and defined by Cisco Systems, Inc. as a messaging system between a Skinny

* client and the Cisco CallManager. Examples of skinny clients include the Cisco 7900 series of IP phones,

* Cisco IP Communicator softphone and the 802.11b wireless Cisco 7920.

*

* Skinny is a lightweight protocol which allows for efficient communication with Asterisk. The SCCP Protocol

* is a lower level protocol when compared to for example SIP. It works on Layer 3/4 of the OSI Layer Model.

*

* The SCCP protocol layer supports connectionless and connection-oriented protocol services,

* and network management procedures are carried out on a per protocol service basis, that is, network management

* procedures at SCCP are independently treated for connectionless and for connection-oriented protocol services.

*

* It uses the Real-time Transport Protocol (RTP) over UDP-transport for the bearer traffic (real-time audio stream)

* with other Skinny clients or an H.323 terminal. SCCP is a stimulus-based protocol and is designed as a

* communications protocol for hardware endpoints and other embedded systems, with significant CPU and memory constraints.

**/